# Building a LiDAR-Equipped Mobile Robot in ROS 2 Jazzy Using Python Packages

## Objective

In this tutorial, you will build a simple mobile robot from scratch, add a simulated 2D LiDAR sensor, launch it in Gazebo Harmonic, and visualize the scan data in RViz.

By the end of this tutorial, you will understand:

- ROS 2 Python packages

- URDF robot modeling

- TF frames

- Gazebo Harmonic sensors

- ROS–Gazebo bridging

- LaserScan messages

- RViz visualization

## Learning Pipeline

URDF → TF → Gazebo Sensor → ROS Topics → RViz

## System Requirements

- Ubuntu 24.04 LTS

- ROS 2 Jazzy Jalisco

- Gazebo Harmonic

Verify your ROS installation:

echo $ROS_DISTRO

Expected output:

jazzy

## Step 1: Install Required Packages

sudo apt update

sudo apt install -y \

python3-colcon-common-extensions \

ros-jazzy-ros-gz \

ros-jazzy-rviz2 \

ros-jazzy-xacro \

ros-jazzy-tf2-tools \

ros-jazzy-robot-state-publisher \

ros-jazzy-joint-state-publisher-gui

Source ROS 2:

source /opt/ros/jazzy/setup.bash

## Step 2: Create a Workspace

Create a dedicated workspace for this project.

mkdir -p ~/lidar_bot_ws/src

cd ~/lidar_bot_ws/src

Your workspace should look like this:

~/lidar_bot_ws/

└── src/

## Step 3: Create a Python Package

Create the package:

ros2 pkg create my_robot_description --build-type ament_python

Move into the package:

cd my_robot_description

Create the required folders:

mkdir -p launch urdf worlds rviz config

Create the Python module directory:

mkdir -p my_robot_description

touch my_robot_description/__init__.py

Your package structure should now look like:

my_robot_description/

├── config/

├── launch/

├── my_robot_description/

│   └── __init__.py

├── resource/

│   └── my_robot_description

├── rviz/

├── urdf/

├── worlds/

├── package.xml

├── setup.cfg

└── setup.py

## Step 4: Configure package.xml

Open the file:

nano package.xml

Add the following dependencies before the closing </package> tag:

<exec_depend>robot_state_publisher</exec_depend>

<exec_depend>rviz2</exec_depend>

<exec_depend>ros_gz_sim</exec_depend>

<exec_depend>ros_gz_bridge</exec_depend>

<exec_depend>xacro</exec_depend>

<exec_depend>joint_state_publisher_gui</exec_depend>

<exec_depend>tf2_ros</exec_depend>

Save and close the file.

## Step 5: Configure setup.py

Replace the contents of setup.py with:

from setuptools import find_packages, setup

from glob import glob

import os

package_name = 'my_robot_description'

setup(

name=package_name,

version='0.0.0',

packages=find_packages(exclude=['test']),

data_files=[

(

'share/ament_index/resource_index/packages',

['resource/' + package_name]

),

(

'share/' + package_name,

['package.xml']

),

(

os.path.join('share', package_name, 'launch'),

glob('launch/*.py')

),

(

os.path.join('share', package_name, 'urdf'),

glob('urdf/*')

),

(

os.path.join('share', package_name, 'worlds'),

glob('worlds/*')

),

(

os.path.join('share', package_name, 'rviz'),

glob('rviz/*')

),

],

install_requires=['setuptools'],

zip_safe=True,

maintainer='your_name',

maintainer_email='your_email@example.com',

description='LiDAR simulation robot for ROS 2 Jazzy',

license='Apache-2.0',

tests_require=['pytest'],

entry_points={

'console_scripts': [],

},

)

## Step 6: Verify setup.cfg

Ensure setup.cfg contains:

[develop]

script_dir=$base/lib/my_robot_description

[install]

install_scripts=$base/lib/my_robot_description

## Step 7: Create the Robot URDF

Create the URDF file:

nano urdf/robot.urdf

Paste the following:

<?xml version="1.0"?>

<robot name="mini_rover">

<link name="base_link">

<visual>

<geometry>

<box size="0.3 0.2 0.1"/>

</geometry>

<material name="blue">

<color rgba="0.2 0.4 0.8 1.0"/>

</material>

</visual>

<collision>

<geometry>

<box size="0.3 0.2 0.1"/>

</geometry>

</collision>

<inertial>

<mass value="2.0"/>

<origin xyz="0 0 0"/>

<inertia

ixx="0.02"

ixy="0.0"

ixz="0.0"

iyy="0.02"

iyz="0.0"

izz="0.02"/>

</inertial>

</link>

<link name="laser_link">

<visual>

<geometry>

<cylinder radius="0.03" length="0.05"/>

</geometry>

<material name="black">

<color rgba="0.1 0.1 0.1 1.0"/>

</material>

</visual>

</link>

<joint name="laser_joint" type="fixed">

<parent link="base_link"/>

<child link="laser_link"/>

<origin xyz="0.1 0 0.08" rpy="0 0 0"/>

</joint>

<gazebo reference="laser_link">

<sensor name="lidar" type="gpu_lidar">

<update_rate>10</update_rate>

<topic>scan</topic>

<gz_frame_id>laser_link</gz_frame_id>

<ray>

<scan>

<horizontal>

<samples>360</samples>

<resolution>1</resolution>

<min_angle>-3.14159</min_angle>

<max_angle>3.14159</max_angle>

</horizontal>

</scan>

<range>

<min>0.12</min>

<max>8.0</max>

</range>

</ray>

</sensor>

</gazebo>

</robot>

Verify the URDF:

check_urdf urdf/robot.urdf

Expected output:

Successfully Parsed XML

## Step 8: Create the Gazebo World

Create the world file:

nano worlds/empty.world

Paste:

<?xml version="1.0"?>

<sdf version="1.8">

<world name="default">

<include>

<uri>model://ground_plane</uri>

</include>

<include>

<uri>model://sun</uri>

</include>

<model name="box">

<static>true</static>

<pose>2 0 0.5 0 0 0</pose>

<link name="link">

<collision name="collision">

<geometry>

<box>

<size>1 1 1</size>

</box>

</geometry>

</collision>

<visual name="visual">

<geometry>

<box>

<size>1 1 1</size>

</box>

</geometry>

</visual>

</link>

</model>

</world>

</sdf>

The obstacle box will generate visible laser returns.

## Step 9: Create the Launch File

Create:

nano launch/sim.launch.py

Paste:

from launch import LaunchDescription

from launch.actions import ExecuteProcess

from launch_ros.actions import Node

from ament_index_python.packages import get_package_share_directory

import os

def generate_launch_description():

pkg_path = get_package_share_directory('my_robot_description')

urdf_file = os.path.join(pkg_path, 'urdf', 'robot.urdf')

world_file = os.path.join(pkg_path, 'worlds', 'empty.world')

with open(urdf_file, 'r') as file:

robot_description = file.read()

return LaunchDescription([

ExecuteProcess(

cmd=['gz', 'sim', '-r', world_file],

output='screen'

),

Node(

package='robot_state_publisher',

executable='robot_state_publisher',

parameters=[{

'robot_description': robot_description,

'use_sim_time': True

}],

output='screen'

),

Node(

package='ros_gz_sim',

executable='create',

arguments=[

'-string', robot_description,

'-name', 'mini_rover'

],

output='screen'

),

Node(

package='ros_gz_bridge',

executable='parameter_bridge',

arguments=[

'/clock@rosgraph_msgs/msg/Clock[gz.msgs.Clock',

'/scan@sensor_msgs/msg/LaserScan@gz.msgs.LaserScan'

],

output='screen'

)

])

## Step 10: Build the Workspace

Move to the workspace root:

cd ~/lidar_bot_ws

Build:

colcon build --symlink-install

Source the workspace:

source install/setup.bash

Add it permanently:

echo "source ~/lidar_bot_ws/install/setup.bash" >> ~/.bashrc

Reload:

source ~/.bashrc

## Step 11: Launch the Simulation

ros2 launch my_robot_description sim.launch.py

Gazebo Harmonic should open with:

- Ground plane

- Obstacle box

- Robot model

## Step 12: Verify Topics

Open a new terminal:

source ~/.bashrc

List available topics:

ros2 topic list

Expected output:

/clock

/robot_description

/scan

/tf

/tf_static

Inspect the LiDAR topic:

ros2 topic info /scan

Monitor scan data:

ros2 topic echo /scan

## Step 13: Visualize in RViz

Launch RViz:

rviz2

Configure RViz:

- Fixed Frame → base_link

Add the following displays:

- RobotModel

- TF

- LaserScan

Set the LaserScan topic:

/scan

You should now see laser points reflecting from the obstacle.

## Step 14: Verify the TF Tree

Generate the TF diagram:

ros2 run tf2_tools view_frames

Open the generated PDF:

xdg-open frames.pdf

Expected TF tree:

base_link

└── laser_link

## Understanding the Data Flow

URDF

↓

Gazebo Sensor Plugin

↓

Gazebo Transport

↓

ros_gz_bridge

↓

/scan

↓

RViz

## What You Learned

- Creating ROS 2 Python packages

- Building robots using URDF

- Creating TF trees

- Adding Gazebo sensors

- Bridging Gazebo to ROS 2

- Working with LaserScan messages

- Visualizing sensor data in RViz

This foundation prepares you for:

- Differential drive robots

- Wheel encoders

- IMUs

- SLAM Toolbox

- AMCL

- Nav2

The only difference between this simulation and a real robot is the source of the /scan topic.